Krenite S General Information

Common Name: Fosamine-Ammonium

EPA Number: 352-395

MoDOT Commodity Code: HERB50459

Form: Liquid.

Target: Principle target is brush in unmowed areas and encroaching limbs.

Comments: A broad spectrum herbicide, used as a foliar spray to control most types of brush.

### Safety

- Personal Safety

  - Read the label thoroughly.

  - Wear long sleeved shirt, long pants, impermeable gloves, and adequate eye protection when spraying.

  - Wear apron, full face shield and gloves while mixing.

  - Avoid breathing spray mist or dust from mixing.

  - Thoroughly wash with detergent and water, and wash all outer clothing after each use.

- Environmental Concerns

  - Read the label thoroughly.

  - Take precautions to prevent damage to desirable plants through spray drift or run-off.

  - Excess spray mixture and rinse water from clean-up operations may be disposed of on site or at an approved waste disposal facility.

  - Triple rinse containers, puncture and dispose of in a sanitary landfill.

### Mixing Instructions

Mix for handgun spray at the rate of 2 gallons per 100 gallons of water. The effectiveness can be increased by using approved adjuvants at the recommended rate and 4 ounces of sodium gluconate to the mixture.  Krenite is also labeled for cut stump treatment when mixed at the 50% rate with water.

### Application Methods:

- Primary use handgun spray, broadcast spray only in area that will remain unmowed for one year.

- Use at least 100 to 200 psi pressure in the handgun.

- Spray until the foliage is completely wet, but avoid excess run-off.

- For best results apply from July to first fall coloration.

- For side trimming trees or brush, spray only portion of plant to be trimmed.

Go to http://www.cdms.net/LabelsMsds/LMDefault.aspx for label information