TOP TEN SPILL REPORTING AND PREVENTION

MoDOT uses numerous products ranging from petroleum-based fuels, asphalt and bridge deck sealants (Pavon) to pesticides to aid in the maintenance of its road system.  Missouri Law requires any petroleum product spill/release to the environment in excess of 50 gallons or any quantity entering a water body be reported to the Missouri Department of Natural Resources and the National Response Center.  However, MoDOT wants to take a proactive approach to tracking and evaluating spills/releases below reportable quantities by regulation on a statewide basis.  MoDOT can then determine if there are any problem categories where procedures or training needs to be added or modified.

Products most frequently used by maintenance lots are listed below, along with the quantities of  spills/releases outside of secondary containment that are to be reported to the district hazardous materials coordinator (HMC) and MoDOT environmental staff.  Since no one holds a comprehensive list of products used at MoDOT maintenance lots and federal law may require reporting a spill/release of as little as 1 pound of some substances, the HMC and MoDOT environmental staff shall also be contacted for any spills of chemicals not included in this list.  Maintenance personnel for each lot shall keep a record of these spills/releases.  At the end of the calendar year the district HMC shall submit the records to the MoDOT environmental staff.

Since some spills need not be reported to regulatory agencies, it is prudent to consult with MoDOT’s environmental staff before reporting spills to Missouri Department of Natural Resources and/or the National Response Center.    Contact information for the MoDOT environmental staff can be found in the individual facility’s Spill, Prevention, Control and Countermeasures (SPCC) Plan.

Category/Product					Reporting Quantity

- Motor Fuels – Gas/Diesel				5 gallons

- Motor Oils – Engine/Transmission/Hydraulic		5 gallons

- Liquid Asphalt Products 					5 gallons

- Bridge Deck Sealers – Pavon etc.			5 gallons

- Water Base Traffic Paint					5 gallons

- Solvents – Mineral Spirits/Degreasers			1 gallon

- Plant Growth Regulators – Plateau			1 gallon concentrate

5 gallon mixture

- Noxious Weed Control – Garlon 3			1 gallon concentrate

5 gallon mixture

- Brush Control – Milestone/Roundup			1 gallon concentrate

5 gallon mixture

- Stump Control – Basil Oil				1 gallon concentrate

5 gallon mixture