Cathodic System Evaluation

| Date: 8-1-06 Z o n e # | Section A |  |  |  |  |  | Section B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Rectifier Settings |  |  | Ref. Cell and Probe |  |  | Depolarization Readings |  |  |
|  | Input Voltage | Input Current | Input Current Density | On Potential (millivolt) | Instant Off Potential | Rebar Probe | 4 Hr. Off Potential (millivolt) | Shift in Potential (millivolts) | Revised Current (amps) |
| 1 |  |  |  |  |  |  |  |  |  |
| 2 |  | 2.5 | 1.67 | -450 | -410 |  | -285 | -125 |  |
| 3 |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |
| 5 |  | 1.6 | 1.07 | -520 | -490 |  | -380 | -110 |  |
| 6 |  | 1.7 | 1.36 | -180 | -140 |  | -50 | -90 |  |
| 7 |  | 1.7 | 1.36 | -390 | -372 |  | -340 | -32 |  |
| 8 |  |  |  |  |  |  |  |  |  |
| 9 |  | 1.8 | 1.44 | -400 | -385 |  | -285 | -100 |  |
| 10 |  | 1.4 | 1.12 | -315 | -285 |  | -190 | -95 |  |

Name:   John Rust

Bridge Number: 	A1527	Dir: E&W

Route:                      County:

Route	I-222	County	Royals

Location:  Over Blue River

Rectifier Serial:  XJ5-4172

Rectifier Brand Name:   PowerPlus

Air Temperature:	83

Date Let:

Deck Condition:  W                  5/27/1987

Type Anode:    Platinum

Cathodic System

(Mark Yes or No)

Broken Lead:

Zone Loc:

Exposed Lead:

Comment:

Make comments on overall appearance of the system or any other problems.

Corroded Lead:                               Yes

Meters Not  Functioning:	          Yes

Fuses Blown:

Corroded Circuit Board:                  Yes

Lightening Arrestors Blown

Power Off

Lost ID Tags

Are all zones on constant                No	--------->	If not, list zones on constant voltage                  7

Meter Type

Engineer’s comments from main database on Section C, changes in original set up of system, or changes made in operating parameters when new input currents set.

Comments:

External Meter:     Yes

Built In Meter:

| Z o n e # | Section C |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Original Settings |  |  |  | Fixed Design Parameters |  |  |  |
|  | Original On Potential | Original Input Current | Zone Size (Sq. Ft.) | Input Current Density (ma/sf) | Current Parameters |  | Potential Parameters |  |
|  |  |  |  |  | Lower | Upper | Lower | Upper |
| 1 | -550 | 2.25 | 1500 | 1.50 | 0.75 | 3.00 | -330 | -770 |
| 2 | -450 | 3.00 | 1500 | 2.00 | 0.75 | 3.00 | -270 | -630 |
| 3 | -605 | 2.30 | 1500 | 1.53 | 0.75 | 3.00 | -363 | -847 |
| 4 | -511 | 1.80 | 1500 | 1.20 | 0.75 | 3.00 | -307 | -715 |
| 5 | -520 | 1.40 | 1500 | 0.93 | 0.75 | 3.00 | -312 | -728 |
| 6 | -180 | 2.25 | 1250 | 1.80 | 0.63 | 2.50 | -108 | -252 |
| 7 | -390 | 0.50 | 1250 | 0.40 | 0.63 | 2.50 | -234 | -546 |
| 8 | -322 | 2.00 | 1250 | 1.60 | 0.63 | 2.50 | -193 | -451 |
| 9 | -400 | 1.70 | 1250 | 1.36 | 0.63 | 2.50 | -240 | -560 |
| 10 | -315 | 2.40 | 1250 | 1.92 | 0.63 | 2.50 | -247 | -575 |