Metsulfuron-Methyl DF General Information

Common Name: Metsulfuron-methyl

EPA Number: 74477-2

MoDOT Commodity Code: 6758542440(8 oz.)

Form: Dry granules to be dispersed in water.

Target: Used in combination with Plateau to control weeds and brush, such as around guardrails and post, and to inhibit growth and seed head formation in grass(PGR). Used to spot treat brush in mowed areas.

Comments: Metsulfuron-Methyl DF is a post-emergent, broad spectrum herbicide, applied as a foliar spray.

### Safety

- Personal Safety

  - Read the label thoroughly.

  - Wear long sleeved shirt, long pants, impermeable gloves, and adequate eye protection when spraying.

  - Wear apron, full face shield and gloves while mixing.

  - Avoid breathing spray mist or dust from mixing.

  - Thoroughly wash with detergent and water, and wash all outer clothing after each use.

- Environmental Concerns

  - Read the label thoroughly.

  - Do not apply directly to water.

  - Take strong precautions to avoid spray drift or runoff onto desirable plants.

  - Dispose of excess spray mixture and rinse water from cleaning operations in accordance with hazardous waste disposal regulations.

  - Triple rinse containers, puncture and dispose of in a sanitary landfill.

### Mixing Instructions

Metsulfuron-Methyl DF, when tank mixed with Plateau, should be mixed for an application rate of 1/4 to 1/3 ounce per acre. This tank mix will require vigorous and continuous agitation.  A suspension agent must be added if the product is allowed to sit for an extended period of time (overnight).  Use approved adjuvants at the recommended rate.

### Application Instructions

- Broadcast spray only, do not use handgun when used as growth regulator.

- Use handgun to spot treat brush in mowed areas.

- Thorough wetting is necessary.

- Do not allow spray drift over desirable plants.

- Do not exceed recommended rates.

Maintain continuous agitation while spraying.

# LABEL								MSDS