Perspective General Information

Chemical Name: aminocyclopyrachlor and chlorsulfuron

EPA Number:

MoDOT Commodity Code: 6758542256

Form: New in 2011 – grandular form

Target:  Broadleaf vegetation control, and in combination with Plateau in a PGR mixes.  Product will help control the late season escapes of milkweed and poison ivy in PGR applications.  Also used to broaden the species control for spotted knapweed control.

Comments: A broad-spectrum post-emergent herbicide. Addition of a non-ionic surfactant increases post-emergent effectiveness. Little to no odor associated with this product.  Perspective is a combination of two herbicides, one old and one new and when placed together increases the effectiveness.

### Safety

Personal Safety

  - Read the label thoroughly.

  - Wear long sleeved shirt, long pants, impermeable gloves, and adequate eye protection when spraying.

  - Wear apron, full face shield and gloves while mixing.

  - Avoid breathing spray mist or dust from mixing.

  - Thoroughly wash with detergent and water, and wash all outer clothing after each use.

Environmental Concerns

  - Read the label thoroughly.

  - Do not apply directly to water.

  - Take strong precautions to prevent spray drift or runoff onto desirable plants.

  - Dispose of excess spray mixture and rinse water from cleaning operations in accordance with hazardous waste disposal regulations.

  - Dispose of empty containers in a sanitary landfill.

### Mixing Instructions

For broadcast spraying in mix for a PGR application at a rate of 4.75 ounces per acre mixed with 3 ounces of Plateau.  For handgun spraying, mix 4.75 ounces per 100 gallons of water. Spray mixes containing Perspective require continuous agitation. Use approved adjuvants at the recommended rate.  A suspension agent must be added if the product is allowed to sit for an extended period of time (overnight).

### Application Instructions

- Exercise caution to prevent contact with desirable plants through spray drift or runoff.

- For best pre-emergent results, apply shortly before weed growth begins.

- Maintain continuous agitation while spraying.

Go to http://www.cdms.net/LabelsMsds/LMDefault.aspx for label information