Table 133.5.3.6.4 Continuous Operations Routes

Type 2 Winter Event:  6 – 12 in. of snow in 24 hours OR ½ to ¾ in. of ice

| PAVEMENT TEMPERATURE RANGE AND TREND | INITIAL OPERATION |  |  |  | SUBSEQUENT OPERATIONS |  |  | COMMENTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pavement surface at time of initial operation | Maintenance action | Salt spread rates |  | Maintenance action | Salt spread rates |  |  |
|  |  |  | Pre-wetted solid salt (lb/ln-mi) | Brine (gal/ln-mi) |  | Pre-wetted solid salt (lb/ln-mi) | Brine (gal/ln-mi) |  |
| Above 32ºF steady or rising | Dry, wet, slush or light snow cover | None, see comments |  |  | None, see comments |  |  | 1) Monitor pavement temperature closely for drops toward 32ºF and below 2) Treat slick patches if needed with pre-wetted salt at 100 lb/lane-mi or with brine at 44 gal/ln-mi; plow if needed |
| Above 32ºF, 32ºF or below is imminent; ALSO 30 to 32ºF, remaining in range | Dry | Apply brine or pre-wetted solid salt | 100 | 44 | Plow accumulation and reapply brine or solid salt as needed | 100 | 44 | 1) If the desired plowing/treatment frequency cannot be maintained, the spread rate can be increased to 200 lb/lane-mi to accommodate longer operational cycles 2) Do not apply brine onto heavy snow accumulation or packed snow |
|  | Wet, slush, or light snow cover | Apply brine or pre-wetted solid salt | 100 | 44 |  |  |  |  |
| 20 to 30ºF remaining in range | Dry | Apply brine or pre-wetted solid salt | 150-200 | 65-87 | Plow accumulation and reapply brine or solid salt as needed | 200 | 87 | 1) If the desired plowing/treatment frequency cannot be maintained, the spread rate can be increased to 400 lb/lane-mi to accommodate longer operational cycles 2) Do not apply brine onto heavy snow accumulation or packed snow |
|  | Wet, slush, or light snow cover | Apply brine or pre-wetted solid salt | 150-200 | 65-87 |  |  |  |  |
| 10 to 20ºF, remaining in range | Dry, wet, slush, or light snow cover | Apply pre-wetted solid salt | 200 |  | Plow accumulation and reapply brine or solid salt as needed | 250 |  | 1) If the desired plowing/treatment frequency cannot be maintained, the spread rate can be increased to 500 lb/lane-mi to accommodate longer operational cycles 2) Liquid calcium chloride may be used for pre-wetting salt at colder temperatures |
| Below 10ºF, steady or falling | Dry or light snow cover | Plow as needed |  |  | Plow accumulation as needed | 250 |  | 1) As pavement becomes slick apply salt/abrasive mix to enhance traction. Salt will have limited melting power in this temperature range. |

Notes:  SALT APPLICATIONS.  (1) Time initial and subsequent salt applications to prevent deteriorating conditions or development of packed and bonded snow--timing and frequency of subsequent applications will be determined primarily by plowing requirements.  (2) Apply salt ahead of traffic rush periods occurring during storm.  PLOWING.  Plow before chemical applications so that excess snow, slush, or ice is removed and pavement is wet, slushy, or lightly snow covered when treated.