Diuron General Information

Chemical Name: Diuron	EPA Number: 9779-318

MoDOT Commodity Code: 6759050166(2.5 gal oz.)

Form: Liquid form as of 1999.  Prior to 1999, dry granules to be dispersed in water. Dropped in 2005

Target:  Total vegetation control, alone or in combination with glyphosate.

Comments: A broad-spectrum herbicide. May be used pre or post-emergent. Addition of a non-ionic surfactant increases post-emergent effectiveness when used alone. Diuron is taken into the roots of plants; therefore, it must rain to activate herbicide.

### Safety

- Personal Safety

  - Read the label thoroughly.

  - Wear long sleeved shirt, long pants, impermeable gloves, and adequate eye protection when spraying.

  - Wear apron, full face shield and gloves while mixing.

  - Avoid breathing spray mist or dust from mixing.

  - Thoroughly wash with detergent and water, and wash all outer clothing after each use.

  - When mixing, wear an adequate dust respirator and goggles to keep dust out of eyes.

- Environmental Concerns

  - Read the label thoroughly.

  - Do not apply directly to water.

  - Take strong precautions to prevent spray drift or runoff onto desirable plants.

  - Dispose of excess spray mixture and rinse water from cleaning operations in accordance with hazardous waste disposal regulations.

  - Dispose of empty sacks in a sanitary landfill.

### Mixing Instructions

For broadcast spraying, mix for an application rate of 8 - 10 pounds dry or 6 – 8 quarts liquid per acre. For handgun spraying, mix 8 - 10 pounds dry or 6 –8 quarts liquid per 100 gallons of water. Spray mixes containing Diuron require continuous agitation. Use approved adjuvants at the recommended rate.  A suspension agent must be added if the product is allowed to sit for an extended period of time (overnight).

### Application Instructions

- Exercise caution to prevent contact with desirable plants through spray drift or runoff.

- For best pre-emergent results, apply shortly before weed growth begins.

- Maintain continuous agitation while spraying.

# LABEL								MSDS