MoDOT’s Work Zone Impact Analysis Spreadsheet Directions

The following steps will describe the process of running the program and review the output information.

Step 1: Number of Lanes, Truck Percentage, Work Zone Capacity, and User Cost.

| MoDOT WORK ZONE IMPACT ANALYSIS SPREADSHEET |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TOTAL NUMBER OF LANES |  |  | 2 | NUMBER OF LANES OPEN |  |  |  |  | 1 |  |  |  |  |  |
|  | TRUCK PERCENTAGE |  |  | 25 | OPEN LANE CAPACITY |  |  |  |  | 1600 |  |  |  |  |  |
|  | TOTAL CAPACITY OF WORKZONE |  |  | 1240 |  |  |  |  |  |  |  |  |  |  |  |
|  | (The capacity value is consistent with MoDOT's recommended value for the chosen lane configuration) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | USER |  | TRUCKS | $22.70 |  |  |  |  |  |  |  |  |  |  |  |
|  | COST |  | CARS | $10.30 |  |  |  |  |  |  |  |  |  |  |  |

The information in the blue section is required for the spreadsheet. When the Total Number of Lanes and Number of Lanes Open fields are filled in, the Total Capacity of Work Zone field is populated based on the MoDOT Work Zone Guidelines capacity limits. For example, if the input is changed from Number of Lanes of “3” lanes and Number Lanes Open of “1” then the Total Capacity of Work Zone will change to 960.  If desired, the Total Capacity of Work Zone value may be changed manually to any value that would more accurately represent the work zone capacity value.

The Total Capacity of Work Zone value is based on vehicles per hour per lane.

The User Cost is currently the Missouri rates.  The User Cost field may be adjusted to better represent user cost for a specific area or state.

Step 2: Hourly Volume

The DEMAND field is a blue required field. The demand information is the hourly volume, which is based on vehicles per hour per lane. The Demand information is normally populated from the Transportation Management System (TMS) database. Data from the TMS is calculated from field data and updated every 1-3 yrs, dependent on roadway type.  The Demand field can be populated by actual field data.

TOTAL ARRIVALS field is a running count of the number of vehicles entering the work zone.

TOTAL DEPARTURES field is a running count of the number of vehicles that leave the work zone based on the maximum capacity.

QUEUED VEHICLES are the difference of vehicle arriving versus departures. This work zone has a capacity of 1,240 vehicles/hour.  At 7:00 a.m., 1,371 vehicles arrive, but only 1,240 vehicles can travel through the work zone. There are 131 vehicles that cannot enter the work zone for that particular hour and must wait their turn. If the number of arriving vehicles is greater than the capacity of the work zone lane closures, then the work zone will see a queue and delay.

QUEUE LENGTH field calculates the length (miles) of the queue based on the percentage of passenger vehicles and trucks. The length of vehicles for this example is 25 feet for passenger vehicles and 50 feet for trucks.

For example at 7:00 a.m. there are 131 queued vehicles of which 25 percent (0.25) are trucks. (131 vehicles)*(0.75 percent cars)*(25 feet per passenger vehicles) + (131 vehicles)*(0.25 percent trucks)*(50 feet per trucks) = (2,456.25 feet + 1,637.5 feet) / (2 lanes of open lane) * (5,280 feet/mile) = 0.39 mile queue.

DELAY field is the calculation of the queued vehicles, capacity, and conversion from hours to minutes. For 7:00 a.m. example, (131 vehicles)*(60 minutes/hour)/(1,240 vehicles/hour) = 6.34 minutes.

| SATURDAY |  | TIME | DEMAND | CAPACITY | TOTAL ARRIVALS | TOTAL DEPARTURES | QUEUED VEHICLES | QUEUE LENGTH | DELAY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | AM | 0:00 | 364 | 1240 | 364 | 364 | 0 | 0.00 | 0.00 |
| 1 |  | 1:00 | 331 | 1240 | 695 | 695 | 0 | 0.00 | 0.00 |
| 2 |  | 2:00 | 296 | 1240 | 991 | 991 | 0 | 0.00 | 0.00 |
| 3 |  | 3:00 | 337 | 1240 | 1328 | 1328 | 0 | 0.00 | 0.00 |
| 4 |  | 4:00 | 395 | 1240 | 1723 | 1723 | 0 | 0.00 | 0.00 |
| 5 |  | 5:00 | 621 | 1240 | 2344 | 2344 | 0 | 0.00 | 0.00 |
| 6 |  | 6:00 | 1024 | 1240 | 3368 | 3368 | 0 | 0.00 | 0.00 |
| 7 |  | 7:00 | 1371 | 1240 | 4739 | 4608 | 131 | 0.39 | 6.34 |
| 8 |  | 8:00 | 1331 | 1240 | 6070 | 5848 | 222 | 0.66 | 10.74 |
| 9 |  | 9:00 | 1095 | 1240 | 7165 | 7088 | 77 | 0.23 | 3.73 |
| 10 |  | 10:00 | 1238 | 1240 | 8403 | 8328 | 75 | 0.22 | 3.63 |
| 11 |  | 11:00 | 1196 | 1240 | 9599 | 9568 | 31 | 0.09 | 1.50 |
| 12 | PM | 12:00 | 1276 | 1240 | 10875 | 10808 | 67 | 0.20 | 3.24 |
| 13 |  | 13:00 | 1216 | 1240 | 12091 | 12048 | 43 | 0.13 | 2.08 |
| 14 |  | 14:00 | 1317 | 1240 | 13408 | 13288 | 120 | 0.36 | 5.81 |
| 15 |  | 15:00 | 1326 | 1240 | 14734 | 14528 | 206 | 0.61 | 9.97 |
| 16 |  | 16:00 | 1310 | 1240 | 16044 | 15768 | 276 | 0.82 | 13.35 |
| 17 |  | 17:00 | 1222 | 1240 | 17266 | 17008 | 258 | 0.76 | 12.48 |
| 18 |  | 18:00 | 1091 | 1240 | 18357 | 18248 | 109 | 0.32 | 5.27 |
| 19 |  | 19:00 | 847 | 1240 | 19204 | 19204 | 0 | 0.00 | 0.00 |
| 20 |  | 20:00 | 859 | 1240 | 20063 | 20063 | 0 | 0.00 | 0.00 |
| 21 |  | 21:00 | 695 | 1240 | 20758 | 20758 | 0 | 0.00 | 0.00 |
| 22 |  | 22:00 | 601 | 1240 | 21359 | 21359 | 0 | 0.00 | 0.00 |
| 23 |  | 23:00 | 492 | 1240 | 21851 | 21851 | 0 | 0.00 | 0.00 |

Step 3: Work Duration and Calculated Data

When calculating, a start time and duration of closure is required. The above example starts at 12:00 midnight and operates all day or a 24-hour operation. The start time and duration cannot total more than a 24-hour day. For example, start time is 8:00 a.m. and the longest duration can only be 16-hour duration.

The results from the spreadsheet are tabulated as the following: MAX DELAY – maximum delay in minutes, AVE DELAY – average delay in minutes, COST ($) – total user cost for the vehicle (passenger vehicle and truck), and MAX QUEUE LENGTH – maximum queue length in miles.

| Start Time | 0 |
| --- | --- |
| Duration of closure | 24 |
| count | 24 |
| short | 0 |
| MAX DELAY | 13.35 |
| AVE DELAY | 4.43 |
| COST ($) | $5,813.65 |
| MAX QUEUE LENGTH | 0.82 |

Step 4: Work Zone Queue Length and Delay Graphs

The final product of the spreadsheet is a graphical representation of the queue and delay of the work zone. The entire week is graphed to see the trends of the work zone concerns of queue and delay.