CONTRAST PAVEMENT MARKINGS

1.0  Description.  This work shall consist of furnishing and installing contrast  pavement markings.  The pavement markings shall consist of a black waterborne acrylic pavement marking paint on to which is painted MoDOT wet reflective pavement markings in accordance with the Wet Reflective Pavement Markings provision or painted markings in accordance with Sec 620.  Contrast pavement marking shall only be used on intermittent markings (skips), on concrete surfaces.  All materials and construction procedures shall meet MoDOT specifications and specifically the applicable requirements listed in Sec 620.40, except as noted herein.

2.0  Material.

2.1  Black Paint.  The black paint used as the contrast marking shall consist of an acrylic waterborne that is compatible with the high build paint used in the wet reflective pavement marking system or acrylic waterborne pavement marking paint depending on the type of marking used for the white skips.

2.2 Wet Reflective Pavement Markings.  The wet reflective pavement marking, if used, shall be in accordance with the Wet Reflective Pavement Markings provision.

3.0   Construction Requirements.

3.1  Contrast markings shall be accomplished by first placing the black paint on the pavement.  The white pavement marking shall then be applied on top of the black paint after the black paint has cured according to manufacturer’s recommendations.  The black paint and the white skips shall be placed according to the following diagram:

3.2  The white skip shall be centered on the black skip such that there will be a 1.5 inch border of black on either side of the white skip.    The white skip should be fully located within the black paint area, ideally with some black both on the leading and trailing edges.  Tolerances for the width and length of the black and white skips shall be in accordance with Sec 620.2.4.2.

3.3  If used, the application of the wet reflective marking shall be in accordance with the Wet Reflective Pavement Markings provision.

4.0  Method of Measurement.  Final measurement will not be made except for authorized changes during construction or where appreciable errors are found in the contract quantity.  The revision or correction will be computed and added to or deducted from the contract quantity.

Where required, measurement of contrast pavement marking will be made to nearest linear foot.

5.0  Basis of Payment.  The accepted quantity of black contrast pavement marking will be paid for at the contract unit price for the pay item as included in the contract.  The white markings will be paid for separately.