| DIMENSION | DIMENSIONAL STANDARDS (ft.) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ITEM | VISUAL RUNWAY |  | NON-PRECISION INSTRUMENT RUNWAY |  |  | PRECISION INSTR. RUNWAY |
|  |  |  |  | E | F |  |  |
|  |  | E | F |  | G | H |  |
| A | Approach of primary surface and approach surface | 250.00 | 500.00 | 500.00 | 500.00 | 1000.00 | 1000.00 |
| B | Approach surface length | 5000.00 | 5000.00 | 5000.00 | 10000.00 | 10000.00 | 10,000* |
| C | Approach surface width @ end | 1250.00 | 1500.00 | 2000.00 | 3500.00 | 4000.00 | 16000.00 |
| D | Approach surface slope | 20:1 | 20:1 | 20:1 | 34:1 | 34:1 | 50:1 |

*	Extends additional 40,000 @ 40:1 slope.

E -	Utility Runways (Facilities expected to serve only small airplanes).

F -	Runways larger than utilities (Facilities expected to serve large airplanes).

G -	Visibility minimums greater then 3/4 mile.

H -	Visibility minimums as low as 3/4 mile.

| DIMENSION | DIMENSIONAL STANDARDS (m) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ITEM | VISUAL RUNWAY |  | NON-PRECISION INSTRUMENT RUNWAY |  |  | PRECISION INSTR. RUNWAY |
|  |  |  |  | E | F |  |  |
|  |  | E | F |  | G | H |  |
| A | Approach of primary surface and approach surface | 76.00 | 152.00 | 152.00 | 152.00 | 305.00 | 305.00 |
| B | Approach surface length | 1 524 | 1 524 | 1 524 | 3 048 | 3 048 | 3 048* |
| C | Approach surface width @ end | 381.00 | 457.00 | 610.00 | 1 067 | 1 219 | 4 877 |
| D | Approach surface slope | 1:20 | 1:20 | 1:20 | 1:34 | 1:34 | 1:50 |

*	Extends additional 12 192 m @ 1:40 slope.

E -	Utility Runways (Facilities expected to serve only small airplanes).

F -	Runways larger than utilities (Facilities expected to serve large airplanes).

G -	Visibility minimums greater then 1.2 km.

H -	Visibility minimums as low as 1.2 km.