CONTRAST PAVEMENT MARKINGS

1.0  Description.  This work shall consist of furnishing and the installation of contrast  pavement markings.  The pavement markings shall consist of a black waterborne acrylic pavement marking paint on to which is painted MoDOT wet reflective pavement markings in accordance with the Wet Reflective Pavement Markings provision.  Contrast pavement marking shall only be used on intermittent markings, (skips) on concrete surfaces.  All materials and construction procedures shall meet MoDOT specifications and specifically the applicable requirements listed in Sec 620.40, except as noted herein.

2.0  Material.

2.1  Black Paint.  The black paint used as the contrast marking shall consist of an acrylic waterborne that is compatible with the high build paint used in the wet reflective pavement marking system.

2.2 Wet Reflective Pavement Markings.  The wet reflective pavement marking shall be in accordance with the Wet Reflective Pavement Markings provision.

3.0   Construction Requirements.

3.1  Contrast markings shall be accomplished by first placing the black paint on the pavement.  The wet reflective marking will then be applied on top of the black paint after the black paint has cured according to manufacturer’s recommendations.  The black paint and the wet reflective paint shall be placed according to the following diagram:

3.2  The white skip shall be centered on the black skip such that there will be a 1.5 inch border of black on either side of the white skip.  There shall also be a border of 3 inches of black at the leading and trailing edges of the white skip.  Tolerances for the width and length of the black and white skips shall be in accordance with Sec 620.2.4.2.

3.3  Application of the wet reflective marking shall be in accordance with the Wet Reflective Pavement Markings provision.

3.4  The completed pavement marking system shall meet the initial retroreflectivity requirements of Sec. 620.2.4.1.

4.0  Method of Measurement.  Final measurement will not be made except for authorized changes during construction or where appreciable errors are found in the contract quantity.  The revision or correction will be computed and added to or deducted from the contract quantity.

Where required, measurement of 4 inch, 6 inch, 8 inch or 24 inch pavement marking will be made to nearest linear foot.  Where intermittent lines are specified, deductions will be made for the gaps in pavement marking.

5.0  Basis of Payment.  The accepted quantity of contrast pavement markings will be paid for at the contract unit price for each of the pay items included in the contract.  Contrast markings shall consist of the black base striping and the white wet reflective striping considered as a unit.