EXHIBIT "B"

Route I-35, Clay County

Job No. J4I0110

ACTUAL COST ESTIMATE

XYZ Pipeline Company

Pipeline Relocation MP 209.8-211.0-East Leg, A.F.E. 2104

1.	Materials

250'	-  12 3/4" O.D. Casing	@	$10.09	2,510

6000'	-  8 5/8" O.D. Line Pipe	@	6.41	38,460

Miscellaneous Material (Vent

Pipe, Seals, Rip Rap, etc.)			  1,250

Freight			    400

Subtotal		42,620

Handling Charge 8%	3,410

Total Material		46,030

2.	Right of Way

200 Rods		@	4.50	900

R/W Agent			   400

Total Right of Way	1,300

3.	Product Loss

5200 gallons			5,044

4.	Company (Direct Labor)			4,000

Employee Benefits - 31.36% of 2b,

4a & 8; (Retirement, Social Security,

Vacation, Sick Leave, etc.)			  1,960

Total Company Direct Labor

& fringes		5,960

5.	Company Vehicle and Equipment (pickup

& stopple equipment truck)			1,200

6.	General Overhead & Administration

@ 23.47% of Items 4 and 5	1,680

7.	Contract Expense

Construction Contract

Haul, string, & lay new pipe and encasement	96,000

Take up 5700' pipe (existing)	7,500

Bevel & recondition	3,000

Stopple Technician	  1,200

Total Contract Expense	107,700

8.	Engineering

Surveying		800

Preliminary Engineering		525

Construction Engineering		   525

Total Engineering	1,850

9.	Credits

Salvage - 5700' of 8 5/8" O.D. Line Pipe		23,940

SUMMARY

(1)	Material		46,030

(2)	Right of Way		1,300

(3)	Product Loss		5,044

(4)	Company Direct Labor		5,960

(5)	Vehicle and Equipment		1,200

(6)	General Overhead & Administration		1,680

(7)	Contract Expense		107,700

(8)	Engineering		1,850

(9)	Credits		-23,940

Total Cost	146,824

4220' of 5800' on Company's private easement

4220' = 72.7% = Commission's Obligation

5800'		106,888

1580' = 27.3% = Company's Obligation

5800'		39,936

EXHIBIT "B"

Route I-35, Clay County

Job No. J4I0110

ACTUAL COST ESTIMATE

XYZ Pipeline Company

Pipeline Relocation MP 209.8-211.0-East Leg, A.F.E. 2104

1.	Materials

76 m	-  319 mm O.D. Casing	@	$32.63	2,480

1829 m	-  216 mm O.D. Line Pipe	@	21.37	39,086

Miscellaneous Material (Vent

Pipe, Seals, Rip Rap, etc.)			  1,250

Freight			    400

Subtotal		43,216

Handling Charge 8%	3,457

Total Material		46,673

2.	Right of Way

200 Rods		@	4.50	900

R/W Agent			   400

Total Right of Way	1,300

3.	Product Loss

19,684 liters			5,044

4.	Company (Direct Labor)			4,000

Employee Benefits - 31.36% of 2b,

4a & 8; (Retirement, Social Security,

Vacation, Sick Leave, etc.)			  1,960

Total Company Direct Labor

& fringes		5,960

5.	Company Vehicle and Equipment (pickup

& stopple equipment truck)			1,200

6.	General Overhead & Administration

@ 23.47% of Items 4 and 5	1,680

7.	Contract Expense

Construction Contract

Haul, string, & lay new pipe and encasement	96,000

Take up 1737 m pipe (existing)	7,500

Bevel & recondition	3,000

Stopple Technician	  1,200

Total Contract Expense	107,700

8.	Engineering

Surveying		800

Preliminary Engineering		525

Construction Engineering		   525

Total Engineering	1,850

9.	Credits

Salvage - 1737 m of 216 mm O.D. Line Pipe		23,940

SUMMARY

(1)	Material		46,673

(2)	Right of Way		1,300

(3)	Product Loss		5,044

(4)	Company Direct Labor		5,960

(5)	Vehicle and Equipment		1,200

(6)	General Overhead & Administration		1,680

(7)	Contract Expense		107,700

(8)	Engineering		1,850

(9)	Credits		-23,940

Total Cost	147,467

1286 m of 1768 m on Company's private easement

1286 m = 72.7% = Commission's Obligation

1768 m		107,264

482 m = 27.3% = Company's Obligation

1768 m		40,203