CONTRAST PAVEMENT MARKINGS HIGH BUILD

1.0  Description.  This work shall consist of furnishing and installing a black waterborne acrylic contrasting pavement marking as a foundation for intermittent markings (skips) and solid lane lines on concrete surfaces.  This work shall be in accordance with Sec 620 and accompanying provisions except as modified herein

2.0  Material.  The black contrast marking shall consist of an acrylic waterborne paint that is compatible with the high build paint used in the wet reflective pavement marking system or acrylic waterborne pavement marking paint depending on the type of marking used for the intermittent markings.

3.0   Construction Requirements.

3.1  The Contrast markings shall be accomplished by placing the black pavement marking on the pavement with the application of the white pavement marking on top of the black marking after the black marking has cured according to manufacturer’s recommendations.  The black contrast marking and the white skips shall be placed according to the following diagram:

3.2  The white skip shall be centered on the black skip such that there will be a 1.5 inch border of black on either side of the white skip.  The white skip shall be located within the black paint area with black on both the leading and trailing edges.  Tolerances for the width and length of the black and white skips shall be in accordance with Sec 620.2.4.2.

3.3  As an alternate to the above, contrast marking shall be accomplished by placing the black pavement marking at the same time as the white pavement marking.  If this alternate is used, the white skip shall be centered on the black skip such that there will be a 1.5 inch border of black on either side of the white skip.  No black is required on either the leading or trailing edges.  Tolerances for the width and length of the black and white markings shall be in accordance with Sec 620.2.4.2.

3.4  For solid lane line pavement markings, the contrast shall be placed as described above.

4.0  Method of Measurement.

4.1  Final measurement will not be made except for authorized changes during construction or where appreciable errors are found in the contract quantity.  The revision or correction will be computed and added to or deducted from the contract quantity.

4.2  Where required, measurement of contrast pavement marking will be made to nearest linear foot.

5.0  Basis of Payment.  The accepted quantity of black contrast pavement marking will be paid for at the contract unit price for the pay item as included in the contract. The white intermittent or white solid markings will be paid for separately.