## Milestone General Information

Chemical Name: Aminopyralid

EPA Number: 62719-537

MoDOT Commodity Code: 675 90 50 080

Form: liquid form

Target:  Broadleaf vegetation control and can be used in combination with Plateau in a PGR mixes.  Product will help control the late season escapes of milkweed and poison ivy in PGR applications.  Also used for control of spotted knapweed control.

Comments: A broad-spectrum post and pre-emergent herbicide. Addition of a non-ionic surfactant increases post-emergent effectiveness. Little to no odor associated with this product.

### Safety

Personal Safety

  - Read the label thoroughly.

  - Wear long sleeved shirt, long pants, impermeable gloves, and adequate eye protection when spraying.

  - Wear apron, full face shield and gloves while mixing.

  - Avoid breathing spray mist or mist from mixing.

  - Thoroughly wash with detergent and water, and wash all outer clothing after each use.

Environmental Concerns

  - Read the label thoroughly.

  - Do not apply directly to water.

  - Take strong precautions to prevent spray drift or runoff onto desirable plants.

  - Dispose of excess spray mixture and rinse water from cleaning operations in accordance with hazardous waste disposal regulations.

  - Dispose of empty containers in a sanitary landfill.

  - Milestone should not be sprayed near soybeans.

### Mixing Instructions

For broadcast spraying in mix for a PGR application at a rate of 5 ounces per acre mixed with 3 ounces of Plateau.  The addition of ¼ - 1/3 oz. of Escort may be used at the operator discretion.  For handgun spraying, mix 5 ounces per 100 gallons of water.  Use approved adjuvants at the recommended rate.

### Application Instructions

- Exercise caution to prevent contact with desirable plants through spray drift or runoff.

- For best pre-emergent results, apply shortly before weed growth begins.

- Continuous agitation while spraying not as critical due to liquid formulation.

Go to http://www.cdms.net/LabelsMsds/LMDefault.aspx for label information