Interstate Maintenance Expectations

Performance Measures and Objectives

# Table 120.1 Pavements

| ## Characteristic | Performance Measures and Objectives (0.1 Mile Segment) |
| --- | --- |
| Mainline Pavement Pass or Fail Only | The mainline pavements shall produce a smooth ride and look good. The smoothness of the mainline pavement will be annually measured in the fall with the ARAN van to determine the average IRI. Visual inspections will be conducted in the fall on randomly selected tenth-mile segments to determine the percent of repaired surface area. The mainline pavements on interstates shall have an IRI rating of less than 100, which is considered to be a pavement in good condition. For a sample area to be visually acceptable, it cannot have more than 20% of repairs (cannot be repaired for more than 20% of its length). Proper crack sealing, transverse and longitudinal, performed appropriately can be considered visually acceptable. Random crack sealing, such as alligator cracks, shall not be performed. |
| Ramp Pavement Pass, Fail or NA | The ramp pavements at all interchanges on the Interstate System shall produce a smooth ride and look good. Visual inspections will be conducted in the fall on randomly selected interchanges to determine the percent of repaired surface areas. For a sample area to be visually acceptable, it cannot have more than 20% of repairs (cannot be repaired for more than 20% of its length). Proper crack sealing, transverse and longitudinal, performed appropriately can be considered visually acceptable. Random crack sealing, such as alligator cracks, shall not be performed. |
| Concrete Pavement Repair Pass, Fail or NA | Concrete pavement repairs shall produce a smooth ride and look good. Full-depth concrete repairs on either concrete or asphalt pavements shall have square corners and be finished neatly with the appropriate surface texture. Partial-depth concrete repairs on concrete pavements shall have square corners and be finished neatly with the appropriate surface texture. Visual inspections will be conducted on randomly selected tenth-mile segments in the fall to determine the number of repairs that produce a smooth ride, look good and are performing as intended. 100% of the repairs shall be considered acceptable. Concrete pavement repairs shall be completed within 14 days of detection. |
| Asphalt Pavement Repair Pass, Fail or NA | Asphalt pavement repairs shall produce a smooth ride and look good. Asphalt pavement repairs shall have milled butt joints and square corners. Rumble strip and longitudinal joint repairs shall be milled at a constant width for the entire length of the repair area and have square corners. Visual inspections will be conducted on randomly selected tenth-mile segments in the fall to determine the number of repairs that produce a smooth ride, look good and are performing as intended. 100% of the repairs shall be considered acceptable. Asphalt pavement repairs shall be completed within 14 days of detection. |

Interstate Maintenance Expectations

Performance Measures and Objectives

# Table 120.2 Shoulders

| ## Characteristic | Performance Measures and Objectives (0.1 Mile Segment) |
| --- | --- |
| Paved Shoulder Pass, Fail or NA | Paved shoulders shall drain water away from the mainline pavement, not be distorted or rutted, not have a drop-off/buildup greater than 0.5 in. and be free of vegetation. Any shoulder edge drop-off between 2-3 in. shall be a 1:1 slope and greater than 3 in. shall be a 1V:3H (3:1) slope. Visual inspections will be conducted on randomly selected interchanges and tenth-mile segments in the fall to determine the percent of linear feet of both the inside and outside shoulder that look good and are performing as intended. 90% of the length of both shoulders shall be considered acceptable. Edge ruts shall be addressed according to the Incident Response Plan. |
| Unpaved Shoulder Pass, Fail or NA | Unpaved shoulders shall drain water away from the mainline pavement, not be distorted or rutted, not have a drop-off of greater 1.0 in. and be free of vegetation. Any outside shoulder edge drop-off between 2-3 in. shall be a 1:1 slope and greater than 3 in. shall be a 1V:3H (3:1) slope. Visual inspections will be conducted on randomly selected interchanges in the spring and fall to determine the percent of linear feet of both the inside and outside shoulder that look good and are performing as intended. 90% of the length of both shoulders shall be considered acceptable. Edge ruts shall be addressed according to the Incident Response Plan. |
| Emergency Crossover Pass, Fail or NA | Emergency crossovers shall drain water away from the mainline pavement, not be distorted or rutted, not have a drop-off of greater 1.0 in. and be free of vegetation growth. Any outside edge drop-off between 2-3 in. shall be 1:1 and greater than 3 in. shall be a 1V:3H (3:1) slope. Emergency crossovers shall be properly signed for use by emergency vehicles only. Emergency vehicle crossovers should not be paved to discourage vehicular use. The side slopes on the crossovers shall be a 1V:10H (10:1) or flatter slope. Visual inspections will be conducted on randomly selected tenth-mile segments in the spring to determine the percent of linear feet of the emergency crossover that look good and are performing as intended. 90% of the length of the crossover shall be considered acceptable. |

Interstate Maintenance Expectations

Performance Measures and Objectives

# Table 120.3 Bridges

| ## Characteristic | Performance Measures and Objectives (0.1 Mile Segment) |
| --- | --- |
| Deck and Approach Slab Pass, Fail or NA | Mainline bridge deck and approach slabs shall be maintained to provide a smooth riding transition from one pavement segment to the next. Randomly selected mainline bridge deck and approach slabs will be annually measured in the fall with the ARAN van to determine the average IRI. Visual inspections will be conducted on the same bridges in the spring to determine the deck rating and percent repaired surface area. Mainline bridge deck and approach slabs shall have an average IRI of less than 140 within 0.1 miles of the bridge, a deck rating of at least 6 and no more than 10% visible patching by surface area. |
| Superstructure and Graffiti Pass, Fail or NA | Superstructure elements of overpass bridges shall be in good repair with an acceptable aesthetic appearance and superstructure elements of mainline bridges shall be in good condition. Visual inspections will be conducted on randomly selected mainline bridges and overpass bridges located within the roadway segment in the spring to identify exposed reinforcing steel, loose concrete and graffiti visible from the interstate roadway. Visual inspections will be conducted on all mainline and overpass bridges in the fall to determine the superstructure rating and paint condition. Mainline bridges and overpass bridges shall be free of visible exposed reinforcing steel, loose concrete and graffiti. Mainline bridges shall have a superstructure rating of at least 5. Mainline bridges and overpass bridges shall have a paint rating of at least fair. Graffiti shall be removed within 30 days of detection. |
| Substructure and Graffiti Pass, Fail or NA | Substructure elements of overpass bridges shall be in good repair with an acceptable aesthetic appearance and substructure elements of mainline bridges shall be in good condition. Visual inspections will be conducted on randomly selected mainline bridges and overpass bridges located within the roadway segment in the spring to identify exposed reinforcing steel, loose concrete and graffiti visible from the interstate roadway. Visual inspections will be performed on all mainline and overpass bridges in the fall to determine the substructure rating. Mainline bridges and overpass bridges shall be free of visible exposed reinforcing steel, loose concrete and graffiti. Mainline bridges shall have a substructure rating of at least 5. Graffiti shall be removed within 30 days of detection. |
| Slope Protection Pass, Fail or NA | Slope protection shall be maintained in order to prevent the spill fills of the structure from eroding and to provide protection from undermining of the bridge abutments. Visual inspections will be conducted on randomly selected mainline bridges and overpass bridges located within the roadway segment in the spring to determine if the slope protection on the bridge looks good and is performing as intended. 90% of the concrete slope protection shall be structurally sound without any broken or misaligned slabs. 100% of slope protection visible from mainline shall be free of vegetation. |

Interstate Maintenance Expectations

Performance Measures and Objectives

# Table 120.4 Traffic Control

| ## Characteristic | Performance Objectives (0.1 Mile Segment) |
| --- | --- |
| Striping Pass or Fail Only | 95% of the striping shall not appear damaged or missing to the driver at a distance of 150 ft. from the vehicle. 95% of the stripe should closely match the original stripe location. Minimum requirements of 150 mill candelas per square meter per lux for white and 125 mill candelas per square meter per lux for yellow. Markings that have failed shall be re-striped within 30 days of detection or no later than the following Memorial Day weekend if damaged by winter operations. |
| Pavement Markings Pass, Fail or NA | 95% of the pavement markings shall not appear damaged or missing to the driver at a distance of 150 ft. from the vehicle. RPMs should be removed from the pavements if the lenses are not being systematically maintained. Markings that were damaged during winter operations should be repaired no later than the following Memorial Day weekend. |
| Signs Pass, Fail or NA | 100% of the signs are visible day and night, undamaged and properly mounted. No sign shall be in service for more than 10 years. Damaged flat sheet signs shall be repaired within 30 days of detection. Damaged structural signs shall be repaired within 45 days of detection. |
| Lighting Pass, Fail or NA | Lighting facilities shall be present, complete, functioning and without any structural damage or corrosion. Light poles within the clear zone shall be on breakaway poles. 100% of the lighting facilities shall function as intended. Damaged facilities shall be repaired within 30 days. |
| Delineators Pass, Fail or NA | 95% of the delineators are visible day and night, undamaged and properly mounted. All signing material shall be diamond grade or better. Damaged delineators shall be repaired within 30 days of detection. |
| Glare Foils and Glare Fence Pass, Fail or NA | 95% of the glare foils and glare fence are undamaged and properly mounted. Damaged glare foils and glare fence shall be repaired within 30 days of detection. |
| Dynamic Message Signs Pass, Fail or NA | Dynamic Message signs shall be fully functional at all times and deliver accurate, current, relevant traveler information such as travel times, incident information and work zone information. Air filters shall be changed or cleaned with compressed air at each inspection. Other consumables shall be inspected and changed as needed. Diagnostic reports shall be run for technical performance. LED pixel outages shall be noted and shall be replaced as necessary so that no signs have unlit pixels. Brush and trees shall be maintained as to keep the visibility of the signs unobstructed. |

Interstate Maintenance Expectations

Performance Measures and Objectives

# Table 120.5 Roadside Features

| ## Characteristic | Performance Objectives (0.1 Mile Segment) |
| --- | --- |
| Guard Cable Pass, Fail or NA | 100% of the guard cable shall be properly aligned, properly tensioned, mounted at the proper height, delineated and functioning as intended. Necessary repairs shall be completed within 5 days for emergency repairs and 7 days for routine repairs. |
| Guardrail Pass, Fail or NA | 90% of each section of guardrail shall be properly aligned, mounted at the proper height, delineated and functioning as intended. No more than 20% of the front surface of the railing shall show signs of corrosion. Necessary repairs shall be completed within 5 days for emergency repairs and 10 days for routine repairs. |
| Sound Walls, Retaining Walls and Graffiti Pass, Fail or NA | 100% of the sound walls and retaining walls are structurally sound and free of graffiti that can be seen from the interstate. Graffiti shall be removed within 30 days of detection. |
| Concrete Median Barrier Pass, Fail or NA | 100% of the concrete median barrier shall be in place and free of structural distress. Necessary repairs shall be completed within 14 days of detection. |
| Impact Attenuators Pass, Fail or NA | 100% shall be properly installed, show no signs of major damage and function as intended. Necessary repairs to the attenuator devices shall be completed within 5 days of detection. |
| Fence Pass, Fail or NA | Chain link fence located between the interstate roadway and an outer road shall be functional and free of vegetation and litter. Visual inspections will be conducted on randomly selected tenth-mile segments in the spring to determine if the fencing components look good and are performing as intended. All fences shall be in good repair and functioning as intended. No more than 20% of fabric shall show signs of corrosion or damage. |

Interstate Maintenance Expectations

Performance Measures and Objectives

# Table 120.6 Roadsides

| ## Characteristic | Performance Objectives (0.1 Mile Segment) |
| --- | --- |
| Mowing and Turf Condition Pass or Fail Only | Vegetation height should be maintained between 6 and 12 inches in fully developed areas and between 6 and 18 inches in undeveloped areas. Slopes greater than 1V:3H (3:1) shall not be mowed unless in urban areas with the proper equipment. Transitions between areas shall be gradual and natural. Chemical and/or mechanical trimming should be done as needed so that vegetation height around signposts, delineators, guardrail, guard cable, etc. is consistent with the mowing height. Visual inspections will be conducted on randomly selected tenth-mile segments in the fall to determine if the right of way is being maintained in accordance with the vegetation management policy. Vegetation control may be completed up to at least 30 ft. from the edge of the nearest traveled way with 90% of the areas around objects trimmed or sprayed. 95% of unpaved areas should have proper vegetative cover, not including rock cuts or fill areas constructed predominately of rock. 100% of the right of way should be clear or treated to control noxious weed growth. |
| Trees and Brush Pass or Fail Only | No tree with a diameter greater than 4 in. at breast height or hardscape shall be within 30 ft. of the nearest traveled way unless behind guardrail. All of the right of way shall be free of hazardous, leaning, or dead trees capable of falling onto the roadway. All signs shall be visible for a distance of 500 ft. on the mainline and 250 ft. on the ramps. |
| Debris and Road Kill Pass or Fail Only | Debris and road kill shall be removed from the roadway, shoulders and roadsides to provide a safe road and an attractive roadside. It shall be removed as soon as possible if located on the roadway or a ramp. Debris on the shoulders and within the clear zone shall be removed within 48 hours of notification. Visual inspections will be conducted on randomly selected tenth-mile segments in the spring and fall to determine number of pieces of debris or road kill present. There will be no findings of debris or road kill within the clear zone. |
| Litter Pass or Fail Only | The total amount of litter or debris shall not exceed one five-gallon bucket. |
| Landscape Beds Pass, Fail or NA | 80% of the vegetation is healthy, well maintained and does not pose a safety hazard. |

Interstate Maintenance Expectations

Performance Measures and Objectives

# Table 120.7 Drainage

| ## Characteristic | Performance Measures and Objectives (0.1 Mile Segment) |
| --- | --- |
| Pipes and Culverts Pass, Fail or NA | Pipes and culverts shall be structurally sound and function as intended. They shall be kept clear and free of silt, debris and vegetation. Visual inspections will be conducted on randomly selected tenth-mile segments in the spring to determine if the pipes and culverts are performing as intended. 90% of the cross-sectional area shall be unobstructed and functioning as intended. No visible evidence of roadway settlement shall exist. |
| Inlets and Drains Pass, Fail or NA | Inlets and drains shall be structurally sound and function as intended. They shall be kept clear and free of silt, debris and vegetative growth. Visual inspections will be conducted on randomly selected tenth-mile segments in the spring to determine if the inlets and drains are performing as intended. 90% of the inlet opening and basin outlet opening shall be unobstructed, the unit shall be structurally sound, and the inlet grates and access covers (where applicable) shall be present and functioning as intended. No evidence of ponding shall exist. |
| Curbs and Gutters Pass, Fail or NA | All curb and gutter sections shall be structurally sound and function as intended. They shall be kept clear and free of silt, debris and vegetation. Visual inspections will be conducted on randomly selected tenth-mile segments in the spring to determine if the curbs and gutters are performing as intended. 100% of the gutter length shall be free of obstructions that would impede drainage flow. 90% of the curb and gutter area shall be free of structural distress. No evidence of ponding shall exist. |
| Ditches Pass, Fail or NA | Roadside, outfall and median ditches shall be unobstructed and function as intended. Visual inspections will be conducted on randomly selected tenth-mile segments in the spring to determine if the ditches look good and are performing as intended. 90% of the cross-sectional area shall be unobstructed and functioning as intended. 90% of any paved ditch shall be structurally sound and no portion shall exhibit signs of undermining. No erosion or scour exceeding 12 in. deep and 10 ft. long shall exist in unpaved ditches. No evidence of ponding shall exist in unpaved ditches. |

Interstate Maintenance Expectations

Performance Measures and Objectives

# Table 120.8 Welcome Centers

| ## Characteristic | Performance Measures and Objectives (0.1 Mile Segment) |
| --- | --- |
| Facilities Pass, Fail or NA | Welcome Centers shall be convenient, clean and safe to provide a place for long distance travelers exiting the interstate to rest. Inspections will be conducted on all Welcome Centers in the spring and fall to determine if they look good and are performing as intended. All inspections should score 90% or above to be acceptable. |
| Grounds Pass, Fail or NA | Welcome Centers shall be convenient, clean and safe to provide a place for long distance travelers exiting the interstate to rest. Visual inspections will be conducted on all Welcome Centers in the spring and fall to determine if they look good and are performing as intended. All inspections should score 90% or above to be acceptable. |
| Parking Lots Pass, Fail or NA | Welcome Centers shall be convenient, clean and safe to provide a place for long distance travelers exiting the interstate to rest. Visual inspections will be conducted on all Welcome Centers in the spring and fall to determine if they look good and are performing as intended. All inspections should score 90% or above to be acceptable. |

Interstate Maintenance Expectations

Performance Measures and Objectives

# Table 120.9 Uninterrupted Traffic Flow

| ## Characteristic | Performance Measures and Objectives (0.1 Mile Segment) |
| --- | --- |
| Scheduled Lane Closures Pass, Fail or NA | Travelers will move safely and efficiently through every construction, maintenance, permit and utility work zone, including operations involving the closure of one or more travel lanes. Quality assurance inspections of work zones are conducted on a daily basis by Central Office and district staffs. The Work Zone Quality Circle completes in-depth technical reviews annually. No more than a 15-minute delay on interstates is considered acceptable. 95% of work zones on interstates shall pass the visibility-mobility reviews performed. |
| Unscheduled Lane Closures Pass, Fail or NA | Minimize the delays resulting from incidents and other unplanned closures by assisting emergency responders to the maximum extent possible. Traveler information map and dynamic message boards display accurate information and updated as estimated duration changes. MoDOT response times. Time to clear incidents. Time to clear backups from incidents. Timely public notification through dynamic message boards and accurate traveler information map location. To achieve 15-minute response times on urban interstates and 30-minute response times on rural interstates. Clear incidents as soon as practical. |
| Winter Events Pass, Fail or NA | All interstates shall remain open to at least one lane of traffic in each direction and be restored to a clear condition after the end of the winter event. Visual observations shall be made after each winter event to determine the time needed to restore the interstates to a clear condition. To return the interstates to a clear condition as soon as possible after the end of the winter event. |